# Using the 1784-PKTX and -PKTXD with Windows 95

## Contents

- Quick Start:    First Time Installation

- Application Compatibility

- Updating a Previous Version

- Removing the Plug and Play Driver

- Release Notes

## Quick Start:    First Time Installation

Getting the PKTX Plug and Play (PnP) driver installed is pretty simple.

When you first install a PKTX or PKTXD and boot Windows 95, you'll be prompted for a driver.    If at that time you select "driver from disk provided by hardware manufacturer," simply point Windows towards these driver files when prompted to do so.

If you've already installed a PKTX before receiving this driver, you probably told Windows 95 not to install a driver.    In this case, just run the PKTXUPDT.EXE program provided and restart Windows 95.    The PKTX should be redetected and you should now have the opportunity to select "driver from disk provided by hardware manufacturer."

## Application Compatibility

At this time, the only software package that makes use of the PKTX PnP driver is Rockwell Software's RSLinx communication package.    To use the PKTX or PKTXD under Windows 95, configure RSLinx to make use of the PKTX, then select the Windows 95 RSLinx driver in your application software.    See the RSLinx documentation and the documentation for your application for further details.

Third party developers wishing to make use of either RSLinx or the PKTX PnP driver for communications should contact the Communications Business Unit of Rockwell Software at 414-321-8000.

## Updating a Previous Version

If you received this driver as an update to an older version, follow these instructions:

- Run the version of PKTXUPDT.EXE provided with this new driver.    You probably have an older copy of this program as well, but run the latest version since this may also be updated.

- Restart Windows 95.    The PKTX or PKTXD should be auto-detected and you'll be prompted for a driver.

- Select "Driver from disk provided by hardware manufacturer" and point Windows 95 to the new driver files.

## Removing the Plug and Play Driver

If you're permanently removing PKTX hardware from a computer, you may want to get rid of the old driver files.    To do this:

- Remove all PKTX and PKTXD hardware from your computer.

- Enter Control Panel and run Add/Remove Programs.

- Select Allen-Bradley 1784-PKTX and click the Add/Remove button.    The PKTX uninstall/update wizard will lead you through the rest of the removal process.

## Revision Notes

### Version 1.01, released for testing 08/29/97

- Fixed a problem which could cause a page fault on rebooting a machine with a PKTX in it, at some time after the initial installation of the driver.

### Version 1.00, released for testing 08/08/97

- Initial beta release for Windows 95 PKTX support.